# Network Notes

# How does the net work?

- Applications can run over TCP/UDP by sending application data through TCP (stream) / UDP (datagram) sockets

- Applications can run directly over IP by sending application data through raw IP sockets

- Client starts communication by requesting connection to server

- Server accepts or declines connection establishment from a client

- After connection has been established, no difference between client and server

- Both client and server can terminate an established connection

- DHCP = Dynamic Host Configuration Protocol, assign dynamic IP addresses to devices

  - Dynamic addressing: device can have different IP address every time it connects to network

- Request and obtain an IP address, from the available pool of IP addresses at the DHCP server

  - requesting device get valid connection to the Internet

- Obtain the address of valid DNS server(s)

  - requesting device can contact these servers to translate names into IP addresses

- Obtain a default gateway

  - Requesting device knows where to send packets when no specific routes match destination of packets

- Cannot request next-hop towards some destination d, so that device can forward a packet to d

- Cannot configure a switch inside a network to forward traffic over all available ports

- Cannot obtain IP address of a given domain name

- Cannot request an Internet route to a destination

- IP addresses = point of attachment in the Internet

  - Logical “position” of a networking interface in the network

  - GPS for internet

- NOT a pair of devices that communicate through same network link

- NOT  a physical ID of a piece of hardware connected to a network, like serial number

- NOT a request/connection in Internet like ticket id in serving queue

- Change Wi-Fi you’re connected to -> different IP, different prefix

- Names: easier-to-remember ways to reach networking interfaces

- DNS store mapping of names to addresses, distributed database that can be contacted through DNS servers

  - NOT stored locally by every device connected

- URLs contain addresses, not names

- Names and addresses cannot be used indistinctly

- Host with single network interface, IP address 129.104.89.175, prefix 24

- The address 129.104.89.225 is local to this interface, belongs to same network link

- The address 173.104.89.174 is not local, packets addressed to it need at least a router to be delivered, need to be sent to default gateway

- A router that receives an IP packet can drop it or forward it

- Cannot modify source/destination IP addresses in the packet, re-send packet over interface which it was received, reply to the packet

- Socket = pair (IP address, port)

- Endpoint in a network communication

- NOT a network communication or a process running in a device connected to a network

- TCP (Transmission Control Protocol): reliable transport of packets, provides retransmissions if packets are lost, ensures data arrives to destination in same order sent

  - Cost: establish connection, acknowledge receive, retransmissions, high latency

  - Transport layer responsible for maintaining end-to-end communication between two devices

  - Possible to send data packets through TCP/IP network without any transport protocol

- UDP (User Datagram Protocol): doesn’t provide above, packets may be lost or in wrong order

  - Reordering or loss not critical or can be handled by application layer

  - Less cost

- Other transport protocols: SCTP, DCCP…

# Components of a computer network

- A layer n (n≥2) = software

- Send packet: each layer adds corresponding header, send to lower layer

- Receive packet: each layer processes and removes the corresponding header, sends it to the upper layer

- Header of layer n’ > n = payload

  - Payload not processed

- Asynchronous Transfer Mode (ATM): datalink protocol, originally designed to support voice+data traffic

  - Not an obsolete network protocol

  - Not a transport protocol

- Carries frames of 53 bytes (5 header, 48 payload)

  - Doesn’t have a MTU of 1500 bytes

- In Ethernet network (cable), collision on link when 2 stations transmit a packet at the same time

- Listen while transmit: if transmission other than own received, collision

- 2 stations sometimes know, sometimes don’t whether collision occurred

- Minimal duration of transmission < 2*maximum propagation time: some collisions may be unnoticed

- Collision detected by transmitting stations: involved stations retransmit after a random time delay (not deterministic)

  - New collision: new transmission scheduled, time delay determined randomly over longer interval

  - Interval for retransmission doubles each time collision detected (reduce proba of new collisions) = exponential backoff

  - Third station doesn’t retransmit overheard transmission

- Neither of two stations will detect collision if transmissions too small or stations “too far” (propagation delay too large)

- IPv4: Address Resolution Protocol (ARP) finds L2 address of a network interface with given IP address, and vice versa (reverse ARP)

  - Not with Network Address Translation (NAT): assign a public IP address to private IP addresses in local network

- IPv6: exchange of Neighbor Discovery messages determines L2 addresses, resolve IPv6 addresses of neighbors in a link

  - Determine relationships between neighboring nodes

  - Replace ARP, ICMP

  - Determine whether L2 address of a neighbor changed, still reachable?

- L2 addresses cannot be used as IPv4/IPv6 or vice versa

- Switches don’t store on switching tables mapping between IP and L2 addresses

  - Given L2 address determine which port to send it

- Routers divide network links (broadcast domains)

- Switches divide network segments (collision domains)

- Same collision domain: cables, repeaters, hubs, bridges

- Collision domain = network segments

- Broadcast domains = minimum number of links (could be more depending on prefix configuration)

- Packet structures (sequence of headers/payload): each layer (in descending order) adds header, (in ascending order) removes header

- [Application (L7) | Transport (L4) | Network (L3) | Data link (L2) | Payload]

  - [IP (L3) | Ethernet (L2) | Payload]

  - [TCP (L4) | Ethernet (L2) | Payload]

  - [Ethernet (L2) | UDP (L4) | Payload] (?)

  - [HTTP (L7) | TCP (L4) | IP (L3) | Ethernet (L2) | Payload]

- Cannot exist:

  - [Ethernet | IP | TCP | HTTP | Payload]

  - [IP | Payload]

- Repeaters amplify signal, regenerates signal, retransmits over one port a signal it receives on the other

  - Overcome physical attenuation

  - Allow network segments to grow longer

- Do not decode and forward signal, inspect datalink header of received packet, do intelligent routing

- Same collision domain

- Hubs: multiport repeater, signal from one port goes out all other ports

- Physical layer (L1)

- Bridge: join two network segments

- Forward traffic between segments

  - If src, dst on same link: don’t forward

- Switch: like bridge with multiple ports, can perform error checking before forwarding

- Switch receive frame for destination not in switching table: forward over all ports except the port where frame was received

- Filter, forward packets between links

- Disjoint collision domains

- Link layer (L2)

- Routers: forward data packets along networks

- Usually use IP address

- Network layer (L3)

- TCP/IP doesn’t have Presentation layer (6), Session layer (5) unlike ISO/OSI model

- Application layer (7):

- Ensure communication with another application program on network is possible

- Ensure receiving device is identified, can be reached, ready to accept data

- Enables authentication between devices

- Make sure communication interfaces exist (Ethernet/wifi interface in sender’s computer?)

- Present data on receiving end to user application

- Protocol specific to type of application (file transfer, email…)

- Transport layer (4):

- End-to-end semantics (configuration, negotiation between source and destination of transmission)

- Error detection (via checksums), retransmissions in case of packet loss

- Can handle multiple data streams simultaneously

- NO physical transmission over a channel (cable, air etc)

- NO intermediate forwarding towards destination

- Network layer (3):

- Provide addressing and routing, ensure messages delivered to their destination

- Encapsulate messages from higher layers into datagrams

- IP: attempts to deliver datagrams (IP packets) on best-effort basis

- Routers forward incoming datagrams according to destination IP address specified in packet

- L3 addresses: logical, virtual addresses

- Ex: IP address

- Belong to single protocol

- A computer can have any number of L3 addresses

- Can be found by DNS

- Datalink (layer 2) technologies/protocols: Avian carrier, Ethernet, Token Ring, ATM, Wifi

- Roles of datalink layer:

  - Communication between 2 interfaces on same physical link

  - Error detection (eg via checksums), correction

  - Doesn’t manage end-to-end semantics (retransmissions, send/receive acknowledgements by destination)

  - Doesn’t do “best-effort” forwarding of packet from one link to another

- L2 addresses: also called hardware, link-layer (LL) addresses

  - NOT MAC address

- Pertains to the actual hardware interface in computer

- Need to be unique on a link (devices on same link = same L2 address)

  - Need not be unique on internet

- Hardcoded (not assigned dynamically) into network interface

  - Identify a networking device with 48 bits (6bytes) and vendor

  - Can be changed by user

# DNS

- Name-to-address translation in ARPANET:

- File /etc/hosts (like Yellow Pages) manually maintained at Stanford Research institute by researcher

- File contain tuples (name, host) for all hosts connected in ARPANET

- Manual management: didn’t scale when number of hosts on internet increased

- DNS not in use yet

- DNS:

- Application-layer protocol

- Can run over TCP or UDP

  - Use TCP for zone transfers, when packet size too big

- Mostly run over UDP

  - Much faster

  - DNS requests generally very small, fit in UDP segments

  - Add reliability on application layer

- Fully Qualified Domain Names: end with ‘.’, specify exact location in tree of DNS

- www.polytechnique.edu.

- mail.polytechnique.edu.

- www.lix.polytechnique.edu.

- NOT polytechnique, edu, polytechnique.edu, http://polytechnique.edu, http://www.polytechnique.edu:80

- DNS can be used for:

- Name to address translation, address to name translation

- Load-balancing

  - DNS distribute list of IP addresses in different order when respond to new client (round-robin)

  - Clients direct request to different servers: reduce traffic, distribute requests to domain across servers

- Alias record: redirection action, multiple names can point to same address

- Resource-sharing: multiple records for same domain

- Anything involving matching names and records

- Resource record types:

- A: Address record, map domain name to IPv4 address of computer hosting domain

- AAAA: like A, for IPv6 address

- ALIAS: can’t use CNAME record but can use ALIAS:

# L3 Forwarding, NAT, NAPT, Internet connection sharing

- NAT/NAPT: map one public IP address to port numbers and private IP addresses

- NAT/NAPT divide Internet users into those with “real” Internet connectivity and those restricted to use application that only use outbound (from user network to Internet) or inbound (from Internet to user network) network connections

- Enable Internet connectivity while hiding internal topology of user’s network

- Don’t enhance security or provide firewall functionalities

- Don’t preserve end-to-end principle

  - End-to-end principle: communications protocol operations defined to occur at end-points of communications system, or as close as possible to resource being controlled

  - Net neutrality: ISP refrain from discriminating data on network

- NAT ≠ NAPT

- NAT: 1-to-1 mapping between private and public IP addresses, port kept intact

- NAPT: 1-to-many mapping, multiple private IP address mapped to single (outside) public IP address by using different port numbers

  - Reduce number of IP addresses publicly reachable from Internet

- Packet received at networking layer for given destination:

- Sent through route in routing table with longest matching prefix to packet destination

- Sent through host route in routing table for which destination = exact packet destination if exists

- NOT sent through route with lowest cost or default route in routing table

- NOT dropped if none of networking interfaces match packet destination

  - Send through default route

- Private IP addresses:

- Not forwarded by routers

- Need to be unique in network (link)

  - Cannot be used by several interfaces in same link

  - Need NOT be unique in internet

- Interface with private address CAN communicate with public IP address by way of NAT or NAPT

  - Cannot be initiated by the public address (private addr cannot be routed unambiguously on Internet)

- Private addressing: deploy local networks and local connectivity, without being assigned any public IP addressing space

Destination 125.105.26.4

prefix          cost interface

(1) 125.104.26.0/16    1      eth0
(2) 125.104.26.0/24   10      eth1
(3) 0.0.0.0/0          1      eth0

- Forward through eth0, because it’s the interface in default entry (3) providing minimum cost (no prefix match)

- 125.104.26.0/16: 125.104.0.0 - 125.104.255.255

- 125.104.26.0/24: 125.104.26.0 - 125.104.26.255

- Between dots: 8 bits, after slash: prefix length in bits

Destination: 10.1.5.65

Network        Interface Next Hop
default        e0        10.1.1.2
10.1.1.0 /24   e0        DC
10.1.2.0 /24   e1        DC
10.1.3.0 /25   s0        DC
10.1.4.0 /24   s1        DC
10.1.5.0 /24   e0        10.1.1.2
10.1.5.64 /28  e1        10.1.2.2
10.1.5.64 /29  s0        10.1.3.3
10.1.5.64 /27  s1        10.1.4.4

- Send to s0 (longest prefix): 01000000 vs 01000001

- Laptop connect to IP subnet with prefix 129.104.125.128/25

- Laptop can use: 129.104.125.231, 129.104.125.129, 129.104.125.177

- Cannot use: 129.104.125.255, 129.104.125.34, 129.104.124.129, 192.168.0.1

Host A: 129.104.33.25/24

Host B: 129.104.33.27/16

- A and B can communicate to each other: same network

- Doesn’t need routing

# Spanning tree protocol

- Bridge Protocol Data Unit (BPDU):

- Root ID | Cost | Switch ID | Port ID

- BPDU advertise better path if:

  - Smaller Root ID

  - Tie: smaller cost

  - Tie: smaller switch ID

  - Tie: smaller port ID

- Switch ID: uniquely identify switch on network

- On-link prefix: prefix assigned to link

- Interfaces connected to a link have an on-link prefix

- Hosts use on-link prefix to determine if packet destination on link, or need to be routed beyond corresponding router (gateway)

- Several on-link prefixes can be merged in aggregation/summary prefix of shorter length: efficiently announce prefixes in rest of internet

- Spanning tree protocol: L2 protocol

- Prevent loops on network, find out how switches are organized

- Initialization: each switch assumes themselves is root, creates own BPDU and forward through all ports

  - Switch ID = lowest port ID

  - Root Switch = switch with lowest ID

- Receive better packet: Change root ID to switch ID of better packet, cost +1

- Send new BPDU over port P iff new BPDU better than any received BPDU over P

- Root port: port via switch reaches root in spanning tree (non-root switch)

- Designated port: Port over which switch sent its own BPDU in last SPT round

  - 1 designated port per segment

  - last port to get root?

- Blocked port: doesn’t participate in frame forwarding, discards frames received in corresponding network segment

- Switch receives over port 20 (10,1,50,50), port 21 (30,0,30,32), 22 (30,1,40,41)

- Send over port 21 (10, 1, 20, 21), port 22 (10, 2, 20, 22)

- Switch 1 receives frame from A with destination B

- If switching table of switch 1 has entry with host B, only host B receives packet

- If switching table of switch 1 doesn’t have entry with B, B C D receive packet

- Don’t care about switch 2 table (B not connected to it)

- Destination F: doesn’t exist in network, no entry in switch 1 table, B C D receive packet (don’t drop)

- Traffic received at switch at port P is never forwarded if P is blocked port

  - P is root port, switch is root, P not designated port, P connected to link whose other endpoints is blocked: irrelevant

# Routing and Routing protocols

- Routers report their list of neighbors to their neighbors (1-hop) via HELLO

- Routers report symmetric links between them and their neighbors to every other router in network through LSA

  - Don’t send their full view of network topology (with neighbors nor other routers)

  - Don’t advertise their costs to reach every destination

- Routers compute shortest paths to every possible destination (Dijkstra) based on topology info received through LSA

- Routing protocol used by routers, allow devices running the routing protocol to build routing tables

  - Doesn’t forward packets

- Routing: build paths between sources and destinations

- Forwarding: operation, networking device determines packet received in interface has to be sent through another interface, on which one

- Only routers (not hosts) involved in routing and forwarding

- Not every router has full view of network topology during protocol

- Packets send from same source to same destination doesn’t always follow same path

- Hosts receive routing configuration (default route, default gateway) through DHCP or SLAAC

- Hellos: verify bi-directional connectivity between neighbors

  - never forwarded, send to each neighbor

  - Sent to 1-hop link local multicast destinations

  - include symmetric neighbors and heard neighbors

  - Sent periodically, or triggered by external events

  - Describe info about neighbors of originating router

  - A knows B is bidirectional neighbor if A receives HELLO from B, A listed as “heard” or “symmetric” neighbor of B in HELLO

- LSAs: flooded throughout network, information about topology, announce bidirectional links to other routers

  - Discover global topology: learn local topology of flooder

  - Retransmit exact copy of each LSA received exactly once

  - Don’t contain all network links known by originator at time it was generated

  - Describe links from originating router to its neighbors

  - “heard” neighbors not included (only symmetric)

  - Set of all LSA: describe full view of network topology as recorded by routers at given time

  - Unique in network, identified by ID of origin router + sequence number

  - Dijkstra run by each router over links described in all LSAs received

  - Links may appear in more than one LSA at a time

  - Each router receives a copy of LSA at least once

  - Each router retransmits LSA exactly once (when they first receive it)

- Forwarding Information Base (FIB) derived from Routing Information Base (RIB)

  - Contains the actual rules followed when packet received at incoming interface, select which exiting for packet

- Routing protocol allows each participating router to determine its RIB

- On-link prefix: interface configured to know, all destinations with this prefix on same link as I

  - Determine if given address reachable directly on same link

- Summarization prefix: router representing several destination prefixes through single routing table entry

  - Way for router to combine set of (longer) prefixes received in LSA from other routers, to single routing table entry

  - Reduce size of routing table

- Aggregation prefix: router advertise several destination prefixes through as single LSA

  - Way for router to combine set of (longer) prefixes to advertise as single entry in LSA

  - Reduce number and/or size of advertisements router generates

- Link-stating routing protocol converged, link (D,E) breaks:

- D and E detect link broke, no longer receive HELLO messages from each other

- D and E update HELLOs they generate (remove D,E as symmetric neighbor)

- D,E issue new LSA describing their new local topology (D: {A,G}, E: {F})

- A receive LSA: A recompute shortest path to F, install new routing table to F (via B), send to F through {A, B, C, F}

# BGP, DNS and IP

- Unicast, anycast: communication between exactly 1 sender, 1 recipient

- Unicast: sender, recipient uniquely identified

- Anycast: sender uniquely identified, recipient chosen from unknown set of candidates

- Multicast: 1 sender, set of recipients

  - Recipients are named but unknown to sender

- Multicast addresses different from unicast addresses

  - Operations by router when processing multicast packet (packet replication, transmission over multiple interface) different than unicast (identification of egress interface, forwarding)

  - Routing tables, protocols for multicast different than unicast

- BGP: routing protocol

- Exchange data about internet paths to IP prefixes between groups of interconnected networks (Autonomous System)

- Runs on border routers in each AS

- Classful addressing: original IP addressing scheme

  - IP divided into class A-E

  - Class A: prefix of 8 bits, 2^(32-8) addresses

  - Class B: 16 bits prefix, 2^16 different addresses

- Classless Interdomain Routing (CiDR): allow to use network prefixes of any size

- Use variable number for network and host portions of address

- More flexible allocation of IP addresses

- Internet Assigned Numbers Authority (IANA): main non-profit responsible for allocation of IP addresses

- Delegates for each region a Regional Internet Registry (RIR) allocation of IP address blocks

- 2011: IANA run out of IPv4 unallocated addresses

- RIR level: slightly later

- Dual-stack, transition technologies: accelerate IPv4 to IPv6 transition

- Dual-stack: allow devices to configured in both IPv4, 6

  - Happy eyeballs: speed up connections, try simultaneously IPv6 and IPv4, stay with fastest

- 6to4 and derivation 6rd: transition mechanisms

  - use IPv6 in endpoints, transfer IPv6 over IPv4 infrastructures by tunneling

  - 6rd: algorithmic method to map IPv4 to 6 addresses

- IPv6 traffic higher during weekends: better support at residential environments

- IPv4 -> IPv6:

- Addresses grow from 4 bytes to 16 bytes

- Basic troubleshooting tools (ping, traceroute) roughly same

- IPv6 version of DHCP: DHCPv6

- DNS basically preserved

- IPv6 Neighborhood Discovery performs tasks handled by IPv4 DHCP and ARP

## Practice Exam

- Protocol stack: abstract mechanism to represent complex communicating system

  - Allows heterogenous networks: between sender/receiver, complex multi-hop network across several different link types (WiFi, Ethernet, Optical Fiber…) may exist

  - Lower layers don’t know, don’t care about higher layers, two endpoints may define and use their own protocols without requiring an update to all intermediaries (routers)

  - Provide abstraction: higher layer needs not be aware of what lower layers are used

  - DOESN’T make networks faster because packets are smaller

  - DOESN’T mean only single transport protocol is ever going to be needed

- Protocol: horizontal interaction between 2 instances of same layer

- Layer: well-defined role in communication framework, specific role (task) with specific well-defined responsibilities

  - Each layer interacts with immediately adjacent layers in same stack through send()/receive() primitives

  - Payload at layer (n-1) = (header, payload) from layer n

  - DOES NOT specify finite set of protocols, protocol operations permitted

- Protocol number: allows network layer to identify to which transport layer instance a given data packet should be delivered

- Port number: allows transport layer to uniquely identify an application on computer, determine which application given packet should deliver

  - TCP: 0 reserved, can’t be used, 1 process per (IP, port)

  - UDP: source port is optional, 0 = no port

  - Bind socket (file descriptor) with a transport protocol, IP address and port number

- Application on internet is uniquely identified by (IP address, protocol number, port number)

- Maximum transmission unit (MTU): largest size of packet or frame that can be sent

  - Too large: retransmissions if packet encounters router that can’t handle its size

  - Too small: more header overhead, acknowledgements that have to be sent/handled

  - Layer 2: larger than MTU, silently discarded

  - Layer 3: larger than MTU, fragment (in IPv4)

  - Ethernet: 1518 bytes (1500-bytes payload)

- Minimum transmission unit (mtu):

  - Synonym with minimum frame size

  - Important to detect collisions: enough time for transmitter to get collision signal during its transmission, know it has to retransmit

  - Ethernet: 64 bytes (18-byte header, 46-bytes payload)

- Transport layer: ensure end-to-end properties of communication

- Uniquely ID process by port number

- Offers transport service used by application

- Some protocols provide reliable ordered data delivery, others not same end-to-end semantics

- TCP, UDP transport layer protocols

  - NOT SNMP, SMTP, HTTP

- Different applications on same computer may use different transport layers

- Expects that (inter)networking layer offers (nothing more/less than) end-to-end forwarding of data packets through a multi-hop network

- Demultiplexing construct used to determine to which application a packet is to be delivered, commonly called port number (TCP UDP is port number)

- Can handle flow control, congestion control (adapt transmission rate of sender to capacity of receiver and of intermediate routers on path)

  - Some transport protocols do proactive flow/congestion control, others don’t

- DOES NOT present application data to receiving application

- Network layer provide connectivity across multi-hop network

- Identify unique process by IP addresses + protocol number

- Aka convergence layer

- Network layer protocols distinguished between each other (on same machine) by way of Ethertype

  - Ethertype: field in Ethernet frame (layer 2), indicate which protocol to use in layer 3

  - Almost like port number for layer 2

- Receive unicast L3 datagram: network layer inspects destination address

  - Address identify itself: hand datagram off to appropriate transport layer (determined by protocol number)

  - Other than itself: forward datagram, look up in the FIB interface over which next hop (along shortest path to destination) is reachable, transmit over it

- Forwarding Information Base contain pairs (destination, interface): “to get packet closer to destination, send it over this interface”

  - destination itself or next hop towards destination

  - At most a single entry per destination in FIB

  - Match destination address from incoming diagram and FIB: use longest prefix matching

  - Contain at least destination, cost, next hop

  - May also contain quality of service, interface (i.e eth0)

- Unicast routing table of a router contain:

  - DOESN’T contain path packet will take between router and ultimate destination

  - DOESN’T contain tuple (destination, next-hop, distance, next-hop-queue-length)

  - DOESN’T contain copy of all received control packets (all BGP paths received, or LSAs for OSPF…)

- Routing Information Base (RIB) contain (destination, next-hop, distance): “to get packet closer to destination, if you send it to this next-hop neighbor, then cost of whole path to destination is distance”

  - Zero, one or more entries in RIB

- Best Effort: for incoming diagram, decision can be immediately made to

  - Deliver packet to local process (via transport layer)

  - Forward packet over well-defined and identified interface

  - Silently drop packet

  - DON’T buffer packet or start discovery process to determine path towards destination

  - Makes internet more scalable

  - DOESN’T provide quality-of-service guarantees or reliability

- IPv6 example of network layer protocol

- NOT responsible for providing exclusive access to a communications channel (network access control)

- NOT responsible for providing determined end-to-end semantics to applications

- Routing Protocols (like RIP, OSPF) are NOT network layer protocols

  - RIP runs over UDP, application layer

  - OSPF encapsulated in IP, datalink layer

- ICMP NOT network layer protocol (transport layer)

- In the internet, a router does not:

- Increment TTL field of an IP packet

  - De-incremented by router

- Run protocol such as Spanning Tree Protocol to setup IP routing tables

  - Ran by switches

- Encapsulate incoming IP packet with another

  - IP in IP: add header, done in transport layer

- Fragment IPv6 packet before forwarding it

  - Never in IPv6: packets exceeding MTU are dropped

  - IPv4: Router can break packets into smaller pieces so that fragments can pass through link with smaller MTU (or drop and send ICMP)

- What does a router do?

- Data-link layer: provide transmission of data across a single link

  - NOT against a physical link

- Responsible for Medium Access Control: responsible for moving packets from a Network Interface Card to another across a shared channel

  - Also (framing, error correction ?)

- Identify unique process by HW Address + Ethertype

- DOESN’T need to provide guarantee that data packet handed to Network Layer will be successfully/correctly delivered to intended destination

- 5 collision domains

- 5 broadcast domains

- Minimum number of IP links such that all interfaces are part of a link = 5

- Need minimum of 5 on-link prefixes

- Minimum number of on-link prefixes required = 4